This is a Life Racing Advance Software distribution list email.

Software sent to this list is new and has had minimal testing only. ECU software may not have run on an engine yet. The intent is for you to have advance visibility of builds compared to the general dealer list, which receives builds two weeks later.

Please keep testing in-house for that period, or at least until you have personally verified that everything is working acceptably. Thank you.

ANDY

LifeDash – 2.15.312 to 2.15.348

Warning

  - Any Zero-Offset or item persistence setting will be lost on upgrading to main code 1.5.0

  - Once the value is re-applied, it will be remembered

Features

- Support for new main codes 1.4.20, 1.4.21, 1.5.0, 1.6.x, 1.7.x, 1.8.0, 1.9.0 and 1.9.1

- New on-screen change light modes – 1.9.1

- GPS Beacon detection move from BeaconLine MATH to main code, configured on MISC tab – 1.9.0

- ScreenDimming options added to MISC tab for 1.8.0

- Alignment options for RPM LEDs added to MISC tab for 1.7.3

- "Clear Internal Logging" renamed to "Clear Logging" as now applies to USB as well as MMC for 1.7.1

- Added LifeData functionality to LifeDash for 1.7.0

  - Decoding of LRS files or folder to LRD

  - Option for automatic decoding of downloaded LRS files to LRD

- Option to load latest downloaded LRD file immediately in LifeView

  - Option to support LRD suffix (run number)

  - Option to export LRD data as CSV

- Logging data download or clear now allowed during dev\connect session

- Bugfix – user access update prevented root’s ability to edit alternate units on imported F88 data streams

- Enhanced GPS timeslip for main code 1.6.0

  - Create GPS reference lap from LRD data

  - Add viewer for reference laps

  - Add ability to overlay LRD track data on top of reference lap

  - Add GPS coords of mouse to reference lap viewer

- Enhanced User Access for main code 1.5.0

  - Duplicates LifeCal’s Comment user access

  - Remove user access to

    - No of pages

    - Disable Ethernet and Flash IN-RACE notifications

  - Add fine grained configurable edit access to item attributes on ITEMS tab

  - Prevent users moving read only pages

  - Default all new calibrations to LIMITED user access

Dash Firmware

MAIN CODE

Main Code 1.9.1

Requires boot 4.29

New on-screen change light modes using configurable background colour

Main Code 1.9.0

Requires boot 4.29

GPS beacons setup now in calibration, configured  on LifeDash’s MISC tab

Main Code 1.8.0

Requires boot 4.29

Add new mon items logFileSize and streamFileSize

Add RTC special items for RTC over CAN

Add ScreenDimming mode to MISC calibration

Main Code 1.7.3

Requires boot 4.28

Bugfix – boot loop caused by bug in MMC directory printing (with RTC)

New options to align on-screen RPM LEDs top, middle or bottom

Main Code 1.7.0 / 1.7.1

Requires boot 4.28

Add facility to download USB logging over Ethernet

Main Code 1.6.6

Requires boot 4.26

New monitoring item mmcLogFiles gives total number of internally logged files

Internal logging icon now displays total number of files

bugfix – possible download failure on 2nd download in single LifeDash session

Main Code 1.6.5

Requires boot 4.26

timeSlip - separate GPS and distance based algorithms and only choose based on EnableGpsTimeSlip checkbox on MISC tab

bugfix - only use GPS lat & long to determine if there has been movement

    - removing possibility of AG50.1.speed when set as SPEED triggering current lap update when no GPS lock

Main Code 1.6.4

Requires boot 4.26

Bugfix – prevent timeslip processing when reference lap empty

Fine tune timeslip timeouts for more reliable updates

Main Code 1.6.3

Requires boot 4.26

Increase GPS timeslip updates to 5 a second

Increase timeout on GPS timeslip resyncs to reduce risk of one resync causing another

bugfix - possible div zero could cause spike in timeSlip

Add new monitoring item dbTimeSlipGpsHits - counts timeslip updates (should be 5 a second)

Main Code 1.6.2

Requires boot 4.26

Add On-Screen change lights option (D4 has no LED change lights)

Main Code 1.6.1

Requires boot 4.26

Bugfix – GPS line segments crossing reference lap did not cross track at 90 degrees

Increase GPS line segments to 3 a second (from 2 a second)

Main Code 1.6.0

Requires boot 4.26

Add GPS coords to reference lap, use for more accurate timeSlip and predictedLapTimes

Uses regularly spaced GPS line segments that cross track so time to a set position on track can be accurately timed and compared with a reference lap

Resulting change in calibration structure necessitated new main code version 1.6.x

Main Code 1.5.0

Requires boot 4.26

- Calibration structure changes to support enhanced user access

- Option to disable Ethernet and Flash notifications on a page by page basis

Main Code 1.4.21

Requires boot 4.26

Add support for offloading internal flash logging/streaming data while logging enabled

- Act of initiating Ethernet download suspends logging which is re-instated after download complete or cancelled

- Act of inserting USB memory suspends logging which is re-instated after USB removed

Add support for “clear internal logging” Ethernet command

Add support for streaming logged data directly to PC via LifeData

Add option to disable page tabs

Main Code 1.4.20

Requires boot 4.23

LifeMon compatibility

Bugfix – Enable "Safe to eject USB" popup on DashLogging to USB

Bugfix - Prevent long keypress on OK causing "Safe to eject USB" when no USB or using MMC target

Bugfix - Prevent instances of USBTransferDialog NOT auto cancelling when USB removed

Add new math parser functions : Integral(val, reset) and Interpolate(val, x1,y1, x2,y2, ... )

Main Code 1.4.19

Requires boot 4.22

bugfix - stop navigational keys killing UsbTransferDialog and subsequent logging

Main Code 1.4.18

Requires boot 4.22

bugfix - page tabs on text3x3 and text3x4

new GPS beacons - El Pinar, Autopolis, San Juan, Okayama, Sugo, Everland

Main Code 1.4.17

Requires boot 4.22

Support for Ethernet download of internal flash logging

Main Code 1.4.16

Requires boot 4.21

Increase the maximum number of devices (groups of items) to 64 (from 20)

Add support for linearisation of CAN voltages - ONLY - 5V at 1mV/bit

Main Code 1.4.15

Requires boot 4.21

Disable page tabs when a page change is requested though the CHANGE_PAGE math function

Main Code 1.4.14

Requires boot 4.21

PduX2 added to the named devices able to use the PDU16 page template

Page tabs now show the abbreviated name of current page and neighbouring pages while user manually navigating the screens on the dash – see top section of video below

Main Code 1.4.13

Requires boot 4.21

Add MMC logging logo, used in same position as USB logo

Consistent colour of logging logo (MMC or USB) reporting logging state

Main Code 1.4.12

Requires boot 4.20

First main code capable of logging directly to internal flash memory (MMC) rather than USB.

Can log independently or stream from an LR ECU to MMC or USB

USB_LOGGING_ENABLE MATH function applies to both MMC and USB logging

USB_LOGGING_ENABLE is no longer optional if MMC logging selected

Transfer from MMC to USB only occurs when logging disabled and USB detected

Contents of MMC logging listed in boot messages

Progress dialog for MMC to USB copy

MMC logged data durable and will survive unexpected power outrages with minimal loss of logged data

MMC logging area supports up to 124 individual files spanning up to 1GB of total memory.

BOOT CODE

Boot code 4.29

Required for Main 1.8.0

New SetScreenDimming interface to support main code 1.8.0

Boot code 4.28

Required for Main 1.7.0

exFAT support on USB dropped

Extended USB filesystem support on FAT12/16/32

Boot code 4.27

Required for Main 1.4.21

Screen dimming on filtered voltage on D8 and SD4 to reduce flickering due to noise

Fixed possible watchdog on MMC initialisation

Boot code 4.26

Required for Main 1.4.21

Adds auto LCD dimming to help dash “survive” periods of low power supply voltage

Boot code 4.23

Required for Main 1.4.20

Boot code 4.22

Required for Main 1.4.17+

Add support for Ethernet download of internal logging

Boot code 4.21

Required for Main 1.4.13 to 1.4.16

Updated file system library adds support for exFAT and long file names

Boot code 4.20

Required for Main 1.4.12

Initial boot version supporting Internal flash (MMC) logging